LM60S LED SERIES(Menu Instructions)

- Menu Structure

Main Interface --Setting     ----Run Mode

----DMX Address

----Channel Mode

----Inversion Pan

----Inversion Tilt

----Pan-Tilt Swap

---- Pan-Tilt Encoder

----No DMX Signal

----Display

----Load Default

--Manual

--System    ----Firmware

----DMX Monitor

----System Errors

----Total Fixture Hours

----Partial Fixture Hours

----Total Lamp Hours

--Advanced ----Sensor Monitoring

----Reset Calibration

----Zero Point Correction

----Lamp Life Hours

----Lamp Hours Clear

--English/Chinese

--Screen Rotation

- Setting

| Options | Description |  |
| --- | --- | --- |
| Run Mode | DMX | Status of Slave: receive the DMX signal from the main console or master |
|  | AUTO | Status of Master：Auto Mode and send DMX signal to slave |
|  | Random |  |
|  | Sound |  |
| DMX Address | 1~512 | Press “OK” button into Edit Mode. Now the hundred-bit is selected, press “UP”,”DOWN” to change the address code. Press “OK” again to choose the decade-bit to edit. Press “OK” once again to choose the unit-bit, again to quit the Edit Mode. |
| Channel Mode | Standard 13CH | Standard 13 channel mode |
| Inversion Pan | off |  |
|  | on |  |
| Inversion Tilt | off |  |
|  | on |  |
| Pan-Tilt Swap | off |  |
|  | on | Switch channels of Pan and Tilt axis(including fine-tuning) |
| Pan-Tilt Encoder | on | Use encoder (optocoupler)to estimate the wrong step and right the wrong position |
|  | off | Don’t use encoder(optocoupler) to right the position |
| No DMX Signal | hold | Keep going as this status |
|  | clear | Motor return, stop running. |
| Display | on | Turn off back light after being idle for 30 seconds |
|  | off | Back light always on |
| Load Default |  | The Confirmation dialog will show after pressing “OK” button, press “OK” again to restore the default setting. |

- Manually

This interface is used to control the current lamps, at the same time get into the master status(not receiving DMX signal, send DMX signal to slave through bus.

| =Options | Description |  |
| --- | --- | --- |
| 1CH. Dimmer | 0~255 | Press “OK” button into Edit Mode. Now the hundred-bit is selected, press “UP”,”DOWN” to change the channel value. Press “OK” again to choose the decade-bit to edit. Press “OK” once again to choose the unit-bit, again to quit the Edit Mode. |
| …… | 0~255 |  |
| 12CH.Macro Function | 0~255 |  |
| 13CH.Reset |  | The Confirmation dialog will show after pressing “OK” button, press “OK” again to go into the Restore interface, all the motors reset. |

- System Information

| Options | Descriptions |
| --- | --- |
| Software Version | Current software version |
| DMX Monitor | Go into sub-interface, show the channel value using number and percent |
| System Errors | If red ERR is on, it means lamp runs error, go into sub-interface to check the detail.You can press “Clear” button to clear the log. NOTE: Sometimes, it’s not the problem of the installations of Hall or optocoupler, but the motor lines has been reverted. |
| Total Fixture Hours | Total time of use.(Accurate to minute) |
| Partial Fixture Hours | The time of current use since start up(Accurate to minute) |
| Total Lamp Hours | Time of current lamps on(Accurate to minute) |

| Error | Descriptions |
| --- | --- |
| Reset fail, uart error | The driver board has no response.The line connecting the display board and uart communication lines in driver board., or there’s something wrong with driver board. |
| Pan reset fail | Pan-axis photoelectric switch，or Pan-axis motor’s problem |
| Tilt reset fail | Tilt-axis photoelectric switch，or Tilt-axis motor’s problem |
| Color reset fail | Color Wheel hall, or Color wheel motor’s problem |
| Gobo reset fail | Gobo Hall，or Motor Gobo’s problem |
| lamp hours too long,please replace | Total time of lamp on is over the max value in Advanced setting, to remind the user to replace the lamp. After replace, clear the time value of lamp on in Advanced setting to record from start. |

- Advanced

Set a layer of password here, in case that non-professional misuse. Default password is “UP DOWN UP DOWN”, then “OK”.

| Options |  | Descriptions |  |
| --- | --- | --- | --- |
| Sensor Monitoring | 1234 Hall status | 0-magnet closed, 1-magnet stagger |  |
|  | Pan Code wheel state | 2 digits, each digit correspond to a photoelectric switcher in encoding disk |  |
|  | Tilt Code wheel state | 2 digits, each digit correspond to a photoelectric switcher in encoding disk |  |
|  | Pan Code wheel Step | Positive direction, step value increases, negative direction, step value decreases |  |
|  | Tilt Code wheel Step | Positive direction, step value increases, negative direction, step value decreases. |  |
| Reset Calibration | Pan calibration | X-axis fine tuning | Adjustment range:-128~+127，+0 means no change |
|  | Tilt calibration | Y-axis fine tuning |  |
|  | Color calibration | Color wheel fine tuning |  |
|  | Gobo calibration | Gobo fine tuning |  |
|  | Dimming calibration | Strobe leaf fine tuning |  |
| Zero Point Correction | on | Enable pattern color zero calibration function, adjustment range: +20 |  |
|  | off | Disable pattern color zero calibration function, pattern color doesn’t adjust automatically if out of step |  |
| Lamp Life Hours |  | 0-2000 hours, control the max value will cause the alert |  |
| Lamp Hours Clear |  | After clearing, record from the start |  |